Milestone #2

SDR Design Goals and reasoning:

- Simplicity and reliability

- Low cost and accessibility of components

- Educational demonstrating both analog and digital signal processing

The goal of this project is to design and implement a software-assisted AM radio receiver capable of receiving and demodulating amplitude modulated (AM) signals in the standard broadcast band. The design combines analog front-end circuits with digital processing using a Raspberry Pico to create a flexible and cost-effective receiver.

Reasoning:

- AM modulation was chosen because it is simpler to implement and easier to demodulate compared to FM or SSB.

- A mixer + clock (local oscillator) is used to shift the signal to a lower frequency for easier processing.

- An ADC allows for conversion of analog signals into digital form, enabling software signal processing.

Specifications:

- Frequency range

  - 535–1705 kHz

- Modulation Type

  - AM (Amplitude Modulation)

- Minimum discernible 	signal

  - 10 µV

- User Interface

  - Rotary encoder

  - push buttons

  - LCD/OLED

- Processing Hardware

  - Raspberry Pi Pico RP2040 microcontroller

- Audio Output

  - Headphones or speaker

- Size/Weight

  - Handheld (Walkman size plus antenna)

- Power requirements

  - 5V via USB-C

- Software and computer requirements

  - Nice UI, easy to navigate and makes sense

- Special Features:

  - SD/.WAV recording

  - Storage – SD card or direct over USB

- Cost

  - $40 – $50 depending on components

System Architecture Overview

The receiver follows a Direct Conversion (Zero-IF) architecture. The radio frequency is mixed down to baseband (audio) using a high-speed analog switch driven by a programmable clock.

Signal Path & Sub-Circuits

A. The Front-End (Antenna & Filter)

- Antenna

- Protection: Back-to-back 1N4148 diodes to ground to prevent ESD damage.

- Pre-selector: A Passive Band-Pass Filter (BPF) designed for 500 kHz – 1.8 MHz. This prevents "images" from FM or Shortwave bands from entering the mixer.

- Buffer: A single-transistor JFET buffer (e.g., J310) to match the high-impedance antenna to the low-impedance detector.

- Transformer: Feeds into demodulator.

B. The Demodulator

- Switching IC: A high-speed analog demultiplexer.

- Operation: The switch rotates through 2 positions at the target frequency.

C. The Clock Engine (Digital Tuning)

- Clock Gen: Si5351A controlled via I2C by the Pi Pico.

- Frequency Logic: To listen to 1800 kHz, the Si5351 outputs 1800 kHz.

D. Post-Mixer Amplification (The "Bridge")

- Dual Op-Amp: NE5532 or TL072 configured as differential amplifiers.

- Biasing: Signal is biased for compatibility with the Pi Pico ADC.

- Filtering: Active Low-Pass Filter at 10 kHz to remove switching noise.

E. The Output Split

- Digital Branch: Signal feed into Pico ADC (GP26/27).

- Analog Branch: Signal passes through a 10µF DC-blocking capacitor -> Audio Potentiometer & Power Amplifier -> Speaker/3.5mm Jack.

3. Control & UI (The Pi Pico)

- Input: Rotary encoder for frequency selection.

- Display: I2C OLED to show the current frequency.

- Software: The Pico runs a loop that calculates the frequency for the Si5351 and can optionally perform an FFT on the ADC data to show a "spectrum" of the AM band.

Block Diagram:

Explanation:

- Antenna

  - Receives electromagnetic AM signals from the air.

- AM Filter

  - Filters the incoming signal to isolate the desired frequency range and reduce interference from other signals.

- Mixer + Clock (Local Oscillator)

  - Local Oscillator acts like a signal generator, creating new signals. The mixer then combines the signal, lowering the frequency, making the radio process a lower frequency. Lower frequency allows for easier filtering

- Signal Split Path

  - One path goes to the ADC

  - Another goes through an amplifier for audio output

- ADC

  - Convert the analog signal into digital samples.

- Amplifier and Potentiometer

  - Amplifies audio signal

  - Potentiometer controls volume

- Analog Output

  - Outputs the recovered audio signal thorough speaker or headphones

- Pi Pico

  - Processes the sampled signal

  - Controls display and storage

  - Can demodulation extracting audio from AM signal

- User Interface

  - Encoder: tuning frequency

  - Buttons: controls modes

  - LCD/OLED: display frequency, status

- Power Supply

  - USB-C 5V

- SD card

  - Stores recorded signals or processed data.