SDR Board Bring Up and Testing Plan

Stage 1:  Power Off Check

Verify there are no manufacturing defects

- Visual Inspection: Use magnification to inspect for solder bridges, especially on the PCM1808, 74CBT3253, Si5351, and Pico headers.

Result: PASS

- Continuity Test: Verify there is no short between the 3.3V rail and GND.

Result: PASS, >15k ohm for 5v and >4kohm for 3.3v

- Bias Check: Confirm the 1.65V virtual ground is not shorted to either power or ground. Result: PASS

Stage 2: Power Rail Verification

Verify all supply rails before running the actual circuits or software.

- Initial Power-Up: Apply power using a current-limited bench supply set to approximately 100mA

Result: PASS

- Measurements: Verify the 3.3V rail at the Pico, Si5351, PCM1808, and op amps.

Result: PASS

- Reference Check: Confirm the bias is stable around 1.65V

Result: PASS

Stage 3: Pi Pico & Display

Verify the microcontroller and display operate correctly.

- Software Needed: Simple Pico firmware to initialize the LCD/OLED and display text.

Result: PASS

- Tests: Confirm the display initializes and text appears correctly.

Result: PASS

- Contrast Check: Verify LCD contrast pin Vout is properly grounded.

Result: PASS

Stage 4: I2C Bus Verification

- Software Needed: Pico I2C scanner firmware.

Result: DONE

- Tests: Detect expected device addresses on the bus.

Result: PASS

- Signal Check: Use oscilloscope to confirm SDA and SCL pull-ups and clean transitions.

Result: PASS

Stage 5: Clock Generation & Tayloe Detector

This stage verifies the SDR clocking and quadrature switching system.

- Software Needed: Pico firmware to configure the Si5351 over I2C.

Result: PASS

- Clock Verification: Measure Si5351 outputs and confirm the 74AC74 generates quadrature I_CKL and Q_CLK signals.

Result: PASS

- Expected: Expected: Two stable square waves with one signal shifted 90 degree relative to the other.

Result: PASS

Stage 6: Analog Signal Chain Verification

Before testing RF reception, verify the analog amplification and filtering stages.

- Quiescent Check: Measure op amp outputs and confirm DC bias is approximately 1.65V.

Result: PASS

- Signal Injection: Apply a small 1kHz sine wave from a function generator to the analog input and observe the waveform at each amplifier and filter stage using an oscilloscope.

Result: PASS

- Expected Result: The signal should propagate cleanly through the analog chain with the expected gain and filtering characteristics.

Result: PASS

- Filter Verification: Measure the signal before and after the filter stage to confirm the signal passes through correctly and is modified according to the expected filter response.

Result: PASS

Stage 7: PCM1808 Digital Audio Verification

Verify the ADC and I2S interface operation.

- Software Needed: Pico firmware to capture I2S audio data.

Result: FAIL

- Measurements Verification: Confirm presence of BCK, LRCK, SCKI, and DOUT signals with oscilloscope.

Result: PASS

Stage 8: Full SDR System Test

After verifying all subsystems independently, perform end-to-end SDR testing.

- Connect an antenna or RF signal generator.

Result: PASS

- Tune to a known RF signal.

Result: PASS

- Verify valid I/Q outputs and observable received signals.

Result: PASS

- Confirm the SDR can successfully process and display received data

Result: FAIL